Input Form Instructions

DRL3

Input Form Instructions Version History

| Date | Change Details | Version |
| --- | --- | --- |
| Aug 16, 2007 | Revised | V2 |
| Jan 05, 2016 | Revised | V3 |

## Instructions For the preparation of the Notice of

## Commencement of Drilling (DRL 3)

THE ITEMS WITH THE ASTERISK (*) INDICATE MANDATORY FIELDS.

Name of form

Notice of commencement of drilling.

Objective

To notify the Ministry of Energy and Energy Industries of the existence and location of a new well in the country.

Frequency

As occurs.

Due Date

Within two days following commencement of drilling.

## Copies

Original plus one copy.

Distribution

Chief Technical Officer, c/o Senior Petroleum Engineer, Ministry of Energy and Energy Industries, Maska Building, South Trunk Road, La Romaine or P.O. Box 142 San Fernando.

WELL PLAN INFORMATION

* Operator

Name of company or person operating the lease. Must be in the list of Operator Codes in the Naming Conventions and Standard Codes Document.

* Programme Number

Programme number corresponding to DRL2.  Must be generated using the guidelines in in the Naming Conventions and Standard Codes document (Section 2.41)

* Field

Name of the field in which the well is located. Must be in the list of Field Codes in the Naming Conventions and Standard Codes Document.

* Well Name

Name of the well must be generated using guidelines in Section 2.38 of the Naming Convention and Standard Codes Document.

The well name refers to the full legal name of a well without abbreviations. The Well Name nomenclature is generated using the following rule:

| Field Code |  |  | Dash | Platform Code |  |  | Dash | MEEI's Well Name or Number (Prefix + Number + Hole Types e.g. ST1, X, XST1, etc.) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | N | G | - | C | E | B | - |  |  |  |  |  |  |  |  |  |  |

e.g.

- ANG-CEB-BARAJA_EAST_1 (EXPLORATORY WELLS)

- FOR-LAND-231 (DEVELOPMENT WELLS)

WELL LOCATION: Surface, Objective & Bottom Hole

* Projection

- Type of coordinates e.g. Trinidad Cassini in links, UTM Zone 20 & 21 WGS84 Meters.  Must be in list of Projection Codes in the Naming Conventions and Standard Codes                                   Document.

* Northing Co-ordinate

Location co-ordinates north of the base point.

* Easting Co-ordinate 
Location co-ordinates east of the base point.

* Surface

Surface co-ordinates of the well as accurately as possible at this time.

* Objective Location

Objective co-ordinates of the well as accurately as possible at this time.

* Bottom Hole Location

Bottom hole co-ordinates of the well as accurately as possible at this time.

* Date Drilling Commenced
Date drilling of this well was commenced.

RIG DETAILS

* Rig Type
Type of the drilling rig being used to drill this well. Must be in the list of Rig Type Codes in the Naming Conventions and Standard Codes Document.

* Rig Name
Identifying name of the drilling rig being used to drill this well.

* Contractor
Name of the contractor performing the drilling operations. Must be in the list of Contractor Codes in the Naming Conventions and Standard Codes  Document.

* Contract Type

Whether the contract is for full operations or for labour only. Must be in the list of Contract Type Codes in the Naming Conventions and Standard Codes Document.

DRILLING DURATION

* Estimated Drilling Days
Estimate of rig days required to drill and complete this well.

Ex Well Name
Name of the well at which this rig was last used. Must be generated using the Naming Conventions and Standard Codes Document.

DRILLING REMARKS

Remarks
Any additional information pertinent to the commencement of drilling this well. For marine well, identify (by number or name) the drilling platform and the location on the platform from which this well is being drilled.

CERTIFICATION

*Certification Signature
The duly authorized officer of the company shall sign certifying the statement to be true and correct.

*Certification Date
Date on which the duly authorized officer of the company certified this report. Must be in date format - mm/dd/yyyy.