## Instructions For the Preparation of

## Well Drilling Program (DRL-2)

THE ITEMS WITH THE ASTERISK (*) INDICATE MANDATORY FIELDS.

WELL PLAN PROGRAM

- *Lessee/Owner

Enter name of company or person holding lease.

- *Operator

Enter the name of company or person operating the lease.

- *Program No.

Enter the sequence for the programme number, the programme number will be generated.

- *Pilot Hole

Indicate whether this programme corresponds to a pilot hole

WELL IDENTIFICATION

- *Field

Enter the name of the Field in which the well is located or if no field, enter a short geographical description.

- *Lease Number

Enter the lease number.

- *Well Number

Enter the Well Name if it is an exploratory well, or the Programme Number if it is a development well.

- *Well Classification

Enter a brief description of the well classification, for example, Injection, Observation, or Development; or, if a Lahee well, describe according to the modified Lahee definition of well.

#### *Type of Well

Enter the type of well, Marine, Land –based, Land

#### *Platform

Select the Platform for this well.

WELL LOCATION: SURFACE, OBJECTIVE & BOTTOM HOLE

#### *Projection

Enter the type of coordinates

#### *Surface, Objective (top) and Bottom Hole

Enter the cadastral locations of the well at the depths indicated, as accurately as is possible at this time.

- *Co-ordinates (links)

Enter the location co-ordinates as the number of links north and east of the base point.

ELEVATIONS

21. *R.T. above M.S.L.

Enter the height in feet of the Rotary Table (R.T. above Mean Sea Level (M.S.L.)

##### 22. *Ground level above MSL

Enter the height in feet of the ground at this well site, above Mean Sea Level.  For marine wells quote mean water depth as negative figure.

### PROGRAMMED DEPTHS

24. *Subsea (TVD)

Enter the proposed vertical subsea depth in feet

25. *Drilled (MD)

Enter the proposed measured drilled depth in feet

Boundary Conditions Relating to Contiguous    Lease/Fields

30. *Lease Number

Same as definition above

31. *Distance

Same as definition above

Geological Marker

32. *Horizon

Enter a description of the expected stratigraphy of this well, noting marker depths

drilled and subsea depths of each marker.

33. *Drilled (MD)

Same as definition above

- *Subsea (TVD)

Same as definition above

Instruction for page 2

MUD PROGRAMME HISTORY

Mud Programme

- *Hole Depth

Enter for each depth interval the depths to be given in feet

- *Mud Weight

Enter for each depth interval the weight, the weights in pounds per cubic foot.

- *Mud Type & Composition

Enter for each depth interval the type and composition of the mud to be used.

Distances to Nearest Wells at Objective Horizon

- *Horizon

For each important horizon, markers and depth to be penetrated by the proposed

well

- *Well No.

For each important horizon, markers and depth to be penetrated by the proposed

well, enter in the well no, of each first line well

- *Direction

For each important horizon, markers and depth to be penetrated by the proposed

well, the approximate compass direction

- *Distance

For each important horizon, markers and depth to be penetrated by the proposed

well, enter the approximate horizontal distance in feet of each adjacent well from

the proposed well

Summary of Casing Programme

### *Hole Size

Enter the diameter of the drilled hole in inches

- *Hole Depth

Enter the depth in feet of the well in the indicated hole

###### *Casing Size

Enter the outside diameter (O.D.) of casing in inches

- *Casing Weight

Enter the weight in pounds per linear foot

- *Casing Grade

Enter the grade in standard symbols of the casing set.

- *Casing Shoe At

Enter the depth in feet of the shoe of each string set.

Instruction for page 3

- *Estimated Drilling Cost

Enter the estimated cost of the drilling programme

- *List of attachments to well programme

Enter the attachments being used with the form.